INFRASTRUCTURAL MAPPING OF NITEL QUARTERS AND TELEVISION GARAGE

KADUNA SOUTH LOCAL GOVERNMENT AREA, KADUNA STATE

TABLE OF CONTENTS

CHAPTER ONE	1

1.0	INTRODUCTION	1

1.1	Background of the Study	2

1.2	Infrastructure…………………………………………………………………………….2

1.3	Statement of the Problem	3

1.4	Aim and Objectives	3

1.4.1Aim	3

1.4.2Objectives	3

1.5	Justification	3

1.6	Study Area	4

1.7	Significance of the Project………………………………………………………………6

1.8	Scope of the Project:	6

CHAPTER TWO	8

2.0	LITERATURE REVIEW	8

2.1	Gaps Identified	11

2.2	Summary of Literature Review	11

CHAPTER THREE	12

3.0 MATERIAL AND METHOD	12

3.1	Introduction	12

3.1.1Planning	13

3.1.2	Members of Survey Team	13

3.1.3Reconnaissance	14

3.2	Selection of Station Points	15

3.2.1Pegging		15

3.2.2Station Marking	16

3.3Instrumentation, Test of Instrument and Specification	17

3.3.1Instrumentation	17

3.3.2Test of Instruments	17

3.3.3Field Procedure for Setting Up Differential GPS (Hi-Target V30)	21

3.3.4Procedure for Downloading Data	22

3.3.5Specification	23

3.4	Control Connection and In-Situ Check	23

3.4.1In-Situ Check	23

3.5	Data Acquisition	29

3.5.1Perimeter survey	29

3.5.2Instrumentation and Set-Up	33

3.5.2Materials Used For the Study	39

3.5.3Geometric Data Acquisition	39

3.5.4Attribute Data Acquisition	40

3.6	Database Design	41

3.6.1View of Reality	42

3.6.2Conceptual Design	45

3.6.3Logical Design	45

3.6.4Physical Design	46

3.6.5Construction Phase	46

3.6.6Database Implementation	47

3.6.7Database Management System (DBMS)	47

CHAPTER FOUR	48

4.0DATA PROCESSING, ANALYSIS AND PRESENTATION	48

4.1	Data Processing	48

4.1.1Steps Involved In Downloading Data From Differential GPS (Hi Target V30)Receivers	50

4.1.2Plotting Of Data	51

4.1.3 Detail on CSV to AutoCAD Software	54

4.1.4 Plotting of Details in AutoCAD 2017	54

4.2	Attribute Data	55

4.2.1Spatial analysis	55

4.2.1.2Spatial Search	55

4.2.1.2Single Criteria	56

4.3	Result Presentation and Discussion	56

4.3.1Analysis On Transformers	57

4.3.2. Analysis on Electric Pole	59

4.3.3. Analysis on Road	62

4.3.4. Analysis on Water Facilities	66

4.3.4.1 Borehole Analysis	66

4.3.4.2 Well Analysis	69

4.3.5. Analysis on Social Infrastructure	69

4.3.5.1 Health  Facilities	69

4.4	Data Computation and Analysis of Results	74

4.4.1	Back and Area Computations	74

4.4.2Results Analysis	76

4.5.1Presentation of Data	76

4.5.2Content of the Infrastructure Map	76

CHAPTER FIVE	78

5.0	SUMMARY, CONCLUSION AND RECOMMENDATIONS	78

5.1	Summary	78

5.2	Conclusion	78

5.3	Recommendations	78

REFERENCES	80

APPENDIX	82

LIST OF FIGURES

Figure 1.1: Map of  Nigeria showing Kaduna	5

Figure 1.2: Map of  Kaduna	5

Figure 1.3: Map of Kaduna South LGA showing Television	5

Figure 1.4: Project area boundary	5

Figure 3.1: Flow Chart	13

Figure 3.2: Diagram showing a typical Nail and Bottle cork for monumentation	14

Figure 3.3: Showing the GNSS Satelite, Rover and Base communication	17

Figure 3.4: Showing the control connection	23

Figure 3.5:Showing the description of the GNSS receiver	33

Figure 3.6: Showing the design and construction phase of a spatial database (Kufoniyi, 1998)	36

Figure 3.7:Showing Entity Relationship Diagram(ERD)	37

Figure 3.8:Node entity and its attributes	38

Figure 3.9:Arc entity and its attributes	38

Figure 3.10:Polygon entity and its attributes	38

Figure 4.1: Showing how to add the XYZ downloaded Data (Source: Group 3, 2024)	46

Figure 4.2: Showing the Arc Catalog Window (Source: Group13, 2023)	47

Figure 4.3: Showing how to create a feature class (Source: Group13, 2023)	48

Figure 4.4: Showing the Power Transformer	53

Figure 4.5:Wooden Electric Pole (High Tension) in Good Condition	55

Figure 4.6: Concrete Electric Pole (High Tension) in Fair condition	56

Figure 4.7:Chart Title	59

Figure 4.8:Chart Showing Types of Roads	65

Figure 4.9: Showing the borehole within the Study Area	61

Figure 4.10: Showing Boreholes and Taps	62

Figure 4.11: Showing well in the Study Area	63

Figure 4.12: Showing Primary Heaalth care centers in the Study Area	65

Figure 4.13: Showing Schools…...………………………………………………………………66

Figure 4.14: Attribute Table of Free Land…...…………………………………………………..67

LIST OF TABLES

Table 3.1: Showing the List of Established Control Coordinate	23

Table 3.2: Showing the Coordinate System	24

Table 3.3: Showing the list of Given Coordinates	24

Table 3.4: Showing the list of Observed Coordinates	24

Table 3.5: Showing the differences between observed and computed coordinates	24

Table 3.6: Showing the given and computed bearing and distance for KD08 and CES AP1	25

Table 3.7:Showing the difference between given and computed bearing and distance for CES AP1 and KD08	25

Table 3.8: Showing the perimeter data collected………………………………………………...26

Table 3.9: Showing the detail data collected………………………………………….………....28

Table 3.10:Showing the list of Hardwares used	33

Table 3.11:Showing the list of Softwares used	34

Table 3.12:Showing type of attribute acquired for each feature	34

Table 3.13: Arc entity and its attributes	39

Table 3.14:Polygon entity and its Attributes	39

Table 4.1:Types of Electric Poles……………………………………………………………….57

Table 4.2: Attributes of Road…………………………………………………………………….58

Table 4.3: Classification of Roads……………………………………………………………….58

Table 4.4:Nature of Roads…………………………………………………………………….....60

Table 4.5:Condition of Roads……………………………………………..……………………..60

Table 4.6: Road Drainage System……………………………………………………………….60

Table 4.7: Showing Attribute Table…………………………………………………………..….64

Table 4.8: Attribute of Schools…………………………………………………………………..66

Table 4.9: Attribute of Free Land………………………………………………………………..67

LIST OF PLATES

Plate 3.1: Showing the project menu on the rover	31

Plate 3.2: Showing the device menu on the rover	31

Plate 3.3: Showing the survey menu on the rover	32

Plate 3.4: Showing the GOGO menu on the rover	32

Plate 4.1: Types of transformer…………………………………...……………………………...53

Plate 4.2: Pie chart……………………………………………..………………………………...53

CHAPTER ONE

1.0	Introduction

Historically, surveying has been characterized as the scientific, artistic, and technological practice of determining the relative positions of points above, on, or beneath the Earth's surface. However, Charles O. Ghilani and Paul R. Wolf define surveying more broadly as the discipline that encompasses the gathering and processing of information about the physical Earth and its environment. The Command of Define Council (1968) describes surveying as the scientifically controlled art of measuring the physical features of the Earth and skillfully executing them to delineate a map.

The significance of surveying in shaping the human environment over many centuries is often overlooked. It is a crucial requirement in the planning and execution of nearly every construction project. Surveying has been integral throughout history, contributing to significant scientific discoveries. Its primary modern applications include transportation, construction, land apportionment, and communications. Despite minor differences in technique and the use of handheld instruments, surveying methods are largely consistent worldwide, with instruments primarily manufactured in Switzerland, Austria, Great Britain, the United States, Japan, and Germany.

Infrastructural mapping involves the actual depiction of features on the ground or in the air, facilitated by advancements in photography, aviation, and rocketry. On the ground, surveyors move from one point to another, fixing additional points on features through a combination of angle and distance measurements, ultimately sketching the features freehand. In complex terrain, the differences between ground-based maps and those created from aerial photographs are evident.

Infrastructural mapping is defined as a survey aimed at locating all features on a piece of land, encompassing both natural elements like rocks and trees and artificial features such as buildings and utilities. According to the Command of Define Council (1968), infrastructural mapping is the erratic execution of measuring the Earth's physical features and the scientifically controlled art of delineating them.

The project focuses on the infrastructural mapping of Nitel quarters and the television garage in Kaduna South Local Government Area (LGA), located in Kaduna State, Nigeria. Infrastructural mapping involves the comprehensive survey and depiction of both natural and artificial features on a piece of land, providing a detailed understanding of the existing environment. In the context of Nitel quarters and the television garage in Kaduna South LGA, this project aims to create an accurate and up-to-date map that encompasses various elements, such as buildings, utilities, roads, and natural features.

The choice of NITEL quarters and the television garage in Kaduna South LGA as the project's focus is likely driven by the importance of these locations in the local landscape. NITEL quarters and the television garage are likely to be hubs of activity and central to the community, making an accurate infrastructural map particularly relevant for effective management and development.

The project is expected to involve on-the-ground surveying techniques, potentially utilizing advancements in technology such as aerial photography and satellite imagery interpretation. The resulting infrastructural map will not only serve as a valuable tool for urban planning professionals and local authorities but also contribute to the overall understanding of the region's infrastructure among residents and the broader public.

1.1	Project Location

The study area and extent of project site is in part of NITEL Quarters and Televison Garage in Kaduna South Local Government area of Kaduna state. The total area coverage of the project is 43.1879ha and 431,879.867 square meters and it falls between zone 32 (Minna –Nigeria) of the universal Traverse Mercator projection. Geographically it is located approximately between Latitude10 ˚ 26’ 50” E to 10 ˚ 27’30”N and   Longitude 7 ˚ 25’ 20”N to 7 ˚ 25’  E

1.2	Statement of the Problem

Several interrelated issues underscore the urgency and importance of conducting a comprehensive infrastructural mapping in these areas..

The absence of up-to-date and detailed information regarding the existing infrastructure in NITEL Quarters and Television Garage poses a substantial challenge to urban planners and policymakers.The NITEL Quarters area, havaing undergone a transformation from a telecommunications facility, holds immense potential for redevelopment. Without a detailed understanding of the existing infrastructure, there is a risk of inefficient land use, potentially leading to the underutilization of valuable resources and hindering the optimization of the area for community benefit.

In pursuing these primary objectives of the demarcating, allocating, and settlement of dispute, however this project was made on attempt to identify and forced such problems and make necessary amendment. Also an attempt will be made to deliver whether the perimeter (boundary) survey has been effective.

1.3	Aim and Objectives

1.3.1	Aim

The aim of this project is to produce an infrastructural map of part of NITEL Quarter and Television Garage in Television, Kaduna South Local Government, Showing all the infrastructural facilities (utilities and service) of both public and private owned and to server as information for the distribution and management of infrastructures within the area.

1.3.2 	Objectives

The objectives are as follows:

- To obtain the boundary and the actual area of the study area using differential GPS

- To acquire spatial and attribute data of the existing infrastructural facilities of the study area for proper management of the infrastructures.

- To analyze the spatial distribution of the infrastructures.

- To produce an infrastructural map of NITEL Quarter and Television Garage in Television to serve as information for planners and other authorities.

1.4 	Significance of the Project:

The infrastructural mapping project focusing on Nitel quarters and the television garage in Kaduna South LGA, Kaduna State, Nigeria, holds considerable significance in various aspects. Firstly, accurate and up-to-date infrastructural maps play a pivotal role in urban planning and development. By systematically surveying and mapping the area, the project contributes vital information for policymakers, city planners, and local authorities. This data aids in making informed decisions regarding resource allocation, infrastructure improvements, and sustainable development strategies. Additionally, the project is essential for enhancing disaster preparedness and response by providing a detailed understanding of the existing infrastructure, enabling better planning for emergencies.

1.5	Justification of the Project:

The justification for this infrastructural mapping project is rooted in the need for a Comprehensive understanding of the current state of NITEL quarters and the television garage in Kaduna South LGA. Infrastructural mapping serves as a baseline for future development initiatives and ensures that resources are allocated efficiently. The project addresses the evolving needs of the community, fostering sustainable growth and urban management. Furthermore, the availability of accurate maps aids in attracting investments and promoting economic development by providing potential investors with a clear overview of the area's infrastructure. The project's justification is underscored by its potential to empower local authorities and decision-makers with the necessary information for strategic planning and governance.

1.6	Scope of the Project:

The scope of the infrastructural mapping project extends beyond traditional mapping methods. It encompasses a multidimensional approach, utilizing both on-the-ground surveying techniques and advanced technologies such as aerial photography and satellite imagery interpretation. The project involves mapping natural features, such as land topography, and artificial features, including buildings, utilities, and roads, within the designated area. The scope also involves the identification of potential areas for improvement, whether in terms of infrastructure development, transportation planning, or environmental considerations. By focusing on NITEL quarters and the television garage, the project narrows its scope to areas of particular importance and relevance to the local community, ensuring that the resulting maps are tailored to address the specific needs and challenges of this region.